Andy's Coming! Toy Story AI Robot — Materials Shopping List

Complete 25-Item Bill of Materials  •  Total Estimated Cost: $249.00  •  Verified Active Links

Instructions: Check off each item as you order it. All items below are verified to fit the 3D-printed chassis, 7.4V battery pack, and Raspberry Pi 5 hardware architecture.

1. Compute, Cooling & Storage

| [ ] | Component & Purchase Link | Price | Store | Notes & Specifications |
| --- | --- | --- | --- | --- |
| [ ] | Raspberry Pi 5 (8GB RAM) | $80.00 | Adafruit / CanaKit | Mandatory 8GB RAM for local Gemma LLM + openWakeWord in memory. |
| [ ] | Raspberry Pi Active Cooler | $5.00 | Adafruit / PiShop | Aluminum heatsink + PWM fan; mandatory to prevent CPU throttling. |
| [ ] | SanDisk 64GB/128GB Extreme MicroSD (A2/V30) | $15.00 | Amazon | A2 class rating is critical for high 4KB random IOPS when booting OS and weights. |

2. Vision & Audio Components

| [ ] | Component & Purchase Link | Price | Store | Notes & Specifications |
| --- | --- | --- | --- | --- |
| [ ] | Raspberry Pi Camera Module 3 (Wide 120°) | $35.00 | Adafruit / PiShop | 120° ultra-wide FOV for room/desk inspection. |
| [ ] | 22-Pin to 15-Pin Mini Pi 5 Camera Ribbon Cable (200mm) | $3.50 | Adafruit | MANDATORY: Adapts Pi 5 mini 22-pin CSI pitch to Camera 3 (15-pin). |
| [ ] | INMP441 I2S MEMS Microphone Board | $3.50 | Amazon | Digital I2S interface; zero motor hum for openWakeWord. |
| [ ] | MAX98357A I2S Mono 3W Class D Amp | $4.00 | Adafruit | Digital DAC/Amp directly driven by Pi GPIO pins. |
| [ ] | 4Ω 3W 40mm Enclosed Speaker | $4.00 | Adafruit / Amazon | Fits inside rear torso grill; sealed chamber produces 90s toy resonance. |

3. Locomotion & Neck Actuators

| [ ] | Component & Purchase Link | Price | Store | Notes & Specifications |
| --- | --- | --- | --- | --- |
| [ ] | SG90 9g Analog Micro Servo | $3.00 | Adafruit | Must be ANALOG! 0% PWM duty cycle cuts holding torque for authentic limp drop. |
| [ ] | 2× N20 Micro Metal Gearmotors (6V 100RPM) (1 pair) | $8.00 | Pololu / Amazon | High-torque micro metal gearboxes with 3mm D-shafts. |
| [ ] | DRV8833 Dual H-Bridge Motor Driver | $3.00 | Pololu / Adafruit | Low RDS(on) MOSFETs; VM wired to raw 7.4V battery. |
| [ ] | Pololu 30T Track and Wheel Set (30mm–32mm Wheels) (1 kit) | $14.00 | Pololu / Adafruit | CRITICAL SPEC: 30mm–32mm OD wheels with 3mm D-shaft hubs (2 drive + 4 idlers). |

4. Sensors & Face/Chest Interface

| [ ] | Component & Purchase Link | Price | Store | Notes & Specifications |
| --- | --- | --- | --- | --- |
| [ ] | MPU6050 6-DOF IMU Sensor (I2C) | $3.00 | Adafruit / Amazon | Detects handle lift, shaking, table taps, and cliff edge drops. |
| [ ] | 2× 10mm Diffused Red LEDs + 220Ω Resistors (2) | $1.50 | Adafruit | Jumbo diffused red eyes on GPIO 17; instant black on play dead. |
| [ ] | 2× 6×6mm Panel Tactile Switches (2) | $0.50 | Adafruit | Mounted behind green chest arrow buttons; push to wake/revive. |

5. Power & Battery Management

| [ ] | Component & Purchase Link | Price | Store | Notes & Specifications |
| --- | --- | --- | --- | --- |
| [ ] | 2× 18650 High-Discharge Li-ion (Molicel P28A) (2) | $14.00 | 18650BatteryStore | 2S 7.4V nominal, 35A discharge; prevents voltage sag under CPU/motor loads. |
| [ ] | 2S 18650 Dual Battery Holder with Wire Leads | $2.00 | Amazon | SAFETY: Solderless, spring-loaded series enclosure. |
| [ ] | 2S 10A–15A Li-ion BMS Protection Board | $3.00 | Amazon | Overcharge, low-voltage cutoff (<6.0V), and short-circuit protection. |
| [ ] | 5V 5A High-Efficiency Buck Converter | $5.00 | Pololu / Amazon | Calibrate trim pot to 5.10V before connecting to Pi 5 Pins 2/4. |
| [ ] | Mini SPST Rocker Switch (3A+) | $1.00 | Adafruit | Physical master battery kill switch on rear chassis. |
| [ ] | Type-C 2S 8.4V Li-ion Boost Charger Board | $2.50 | Amazon | Recharges battery pack via standard USB-C phone charger. |

6. Aesthetics, Hardware & Cables

| [ ] | Component & Purchase Link | Price | Store | Notes & Specifications |
| --- | --- | --- | --- | --- |
| [ ] | 80mm Clear Acrylic Fillable Sphere Halves | $3.00 | Amazon | Clear canopy protecting camera lens and glowing red eye LEDs. |
| [ ] | PLA 3D Printing Filament (Blue, Red, Yellow, Green) (1 pack) | $25.00 | Amazon / Polymaker | Multi-color retro toy aesthetic straight from print bed. |
| [ ] | M2.5 & M2 Standoff + Screw Assortment (1 kit) | $8.00 | Amazon | Brass standoffs and screws for PCB and motor mounting. |
| [ ] | DuPont Jumper Wires (40-Pin F-to-F & F-to-M) (1 pack) | $2.50 | Adafruit | GPIO pin-to-sensor interconnections. |

TOTAL ESTIMATED COST: $249.00 USD